SAFETY DATA SHEET

| B BRENNTAG 1. Identification |  |
| --- | --- |
| Other means of identification | None known. |
| Product identifier | SODIUM HYPOCHLORITE 12.5% NSF |
| Recommended use | ALL PROPER AND LEGAL PURPOSES |
| Recommended restrictions | None known. |

Manufacturer/lmporter/Supplier/Distributor information

Manufacturer

| Company name | Brenntag Mid-South, Inc. |  |
| --- | --- | --- |
| Address | 1405 Highway 136, West Henderson, KY 42420 |  |
| Telephone | 270-830-1222 |  |
| E-mail | Not available. |  |
| Emergency phone number 2. Hazard(s) identification | 800-424-9300 CHEMTREC |  |
| Physical hazards | Not classified. |  |
| Health hazards | Skin corrosion/irritation | Category 1 |
|  | Serious eye damage/eye irritation | Category 1 |
| Environmental hazards | Not classified. |  |
| OSHA defined hazards Label elements | Not classified. |  |

| Signal word | Danger |
| --- | --- |
| Hazard statement Precautionary statement | Causes severe skin burns and eye damage. Causes serious eye damage. |
| Prevention | Wash thoroughly after handling. Wear protective gloves/protective clothing/eye protection/face protection. |
| Response | If swallowed: Rinse mouth. Do NOT induce vomiting. If on skin (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower. Wash contaminated clothing before reuse. If inhaled: Remove person to fresh air and keep comfortable for breathing. Immediately call a poison center/doctor. If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| Storage | Store locked up. |
| Disposal | Dispose of contents/container in accordance with local/regional/national/international regulations. |
| Hazard(s) not otherwise classified (HNOC) | None known. |
| Supplemental information | None. |

# 3. Composition/information on ingredients

Mixtures

Chemical name	Common name and synonyms	CAS number

| HYPOCHLOROUS ACID, SODIUM 7681-52-9 SALT (1:1) | 12.5 |
| --- | --- |
| Other components below reportable levels 4. First-aid measures Inhalation Move to fresh air. Call a physician if symptoms develop or persist. | 87.5 |

Skin contact

Eye contact

Ingestion

Most important symptoms/effects, acute and delayed

Indication of immediate medical attention and special treatment needed

General information

# 5. Fire-fighting measures

Suitable extinguishing media

Unsuitable extinguishing media

Specific hazards arising from the chemical

Special protective equipment and precautions for firefighters Fire fighting equipment/instructions

Specific methods

General fire hazards

Take off immediately all contaminated clothing. Rinse skin with water/shower. Call a physician or poison control center immediately. Chemical burns must be treated by a physician. Wash contaminated clothing before reuse.

Immediately flush eyes with plenty of water for at least 15 minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Call a physician or poison control center immediately. Call a physician or poison control center immediately. Rinse mouth. Do not induce vomiting. If vomiting occurs, keep head low so that stomach content doesn't get into the lungs.

Burning pain and severe corrosive skin damage. Causes serious eye damage. Symptoms may include stinging, tearing, redness, swelling, and blurred vision. Permanent eye damage including blindness could result.

Provide general supportive measures and treat symptomatically. Chemical burns: Flush with water immediately. While flushing, remove clothes which do not adhere to affected area. Call an ambulance. Continue flushing during transport to hospital. Keep victim under observation. Symptoms may be delayed.

Ensure that medical personnel are aware of the material(s) involved, and take precautions to protect themselves.

Foam. Powder. Carbon dioxide (C02).

Do not use water jet as an extinguisher, as this will spread the fire.

During fire, gases hazardous to health may be formed.

Self-contained breathing apparatus and full protective clothing must be worn in case of fire.

Move containers from fire area if you can do so without risk.

Use standard firefighting procedures and consider the hazards of other involved materials.

6. Accidental release measures

No unusual fire or explosion hazards noted.

Personal precautions, protective equipment and emergency procedures

Methods and materials for containment and cleaning up

Environmental precautions

# 7. Handling and storage

Precautions for safe handling

Conditions for safe storage, including any incompatibilities

Keep unnecessary personnel away. Keep people away from and upwind of spill/leak. Wear appropriate protective equipment and clothing during clean-up. Do not touch damaged containers or spilled material unless wearing appropriate protective clothing. Ensure adequate ventilation. Local authorities should be advised if significant spillages cannot be contained. For personal protection, see section 8 of the SDS.

Prevent entry into waterways, sewer, basements or confined areas.

Large Spills: Stop the flow of material, if this is without risk. Dike the spilled material, where this is possible. Absorb in vermiculite, dry sand or earth and place into containers. Following product recovery, flush area with water.

Small Spills: Wipe up with absorbent material (e.g. cloth, fleece). Clean surface thoroughly to remove residual contamination.

Never return spills to original containers for re-use. For waste disposal, see section 13 of the SDS. Avoid discharge into drains, water courses or onto the ground.

Do not get in eyes, on skin, or on clothing. Avoid prolonged exposure. Provide adequate ventilation. Wear appropriate personal protective equipment. Observe good industrial hygiene practices.

Store locked up. Store in tightly closed container. Store away from incompatible materials (see Section 10 of the SDS).

# 8. Exposure controls/personal protection

Occupational exposure limits

US. OARS. Workplace Environmental Exposure Level (WEEL) Guide

HYPOCHLOROUS ACID,	STEL	2 mg/m3

SODIUM SALT (1:1) (CAS 7681-52-9)

#:

| Biological limit values Appropriate engineering controls | No biological exposure limits noted for the ingredient(s). Good general ventilation should be used. Ventilation rates should be matched to conditions. If applicable, use process enclosures, local exhaust ventilation, or other engineering controls to maintain airborne levels below recommended exposure limits. If exposure limits have not been established, maintain airborne levels to an acceptable level. Eye wash facilities and emergency shower must be available when handling this product. |
| --- | --- |

Individual protection measures, such as personal protective equipment

The following are recommendations for Personnel Protective Equipment (PPE). The employer/user of this product must perform a Hazard Assessment of the workplace according to OSHA regulations 29 CFR 1910.132 to determine the appropriate PPE for use while performing any task involving potential exposure to this product.

| Eye/face protection Skin protection | Wear safety glasses with side shields (or goggles) and a face shield. |
| --- | --- |
| Hand protection | Wear appropriate chemical resistant gloves. |
| Other | Wear appropriate chemical resistant clothing. |
| Respiratory protection | In case of insufficient ventilation, wear suitable respiratory equipment. |
| Thermal hazards | Wear appropriate thermal protective clothing, when necessary. |
| General hygiene | Always observe good personal hygiene measures, such as washing after handling the material |
| considerations | and before eating, drinking, and/or smoking. Routinely wash work clothing and protective equipment to remove contaminants. |

# 9. Physical and chemical properties

Appearance

| Physical state | Liquid. |
| --- | --- |
| Form | Liquid. |
| Color | CLEAR PALE YELLOW |
| Odor | CHLORINE |
| Odor threshold pH | Not available. |
| Melting point/freezing point | _3 OF (_19.44 0 C) |
| Initial boiling point and boiling range | 212 O F (100 c c) estimated |
| Flash point | Not available. |
| Evaporation rate | Not available. |
| Flammability (solid, gas) | Not applicable. |

Upper/lower flammability or explosive limits

| Explosive limit - lower (%) | Not available. |
| --- | --- |
| Explosive limit - upper (%) | Not available. |
| Vapor pressure | Not available. |
| Vapor density | Not available. |
| Relative density Solubility(ies) | Not available. |
| Solubility (water) | Not available. |
| Partition coefficient (n-octanol/water) | Not available. |
| Auto-ignition temperature | Not available. |
| Decomposition temperature | Not available. |
| Viscosity Other information | Not available. |
| Density | 10.06 lbs/gal 1 .21 g/ml |
| Explosive properties | Not explosive. |
| Oxidizing properties | Not oxidizing. |
| Percent volatile | 87.5 % estimated |

Specific gravity	1 .21

# 10. Stability and reactivity

| Reactivity | Reacts violently with strong acids. This product may react with oxidizing agents. |
| --- | --- |
| Chemical stability | Material is stable under normal conditions. |
| Possibility of hazardous reactions | Hazardous polymerization does not occur. |
| Conditions to avoid | Contact with incompatible materials. Do not mix with other chemicals. |
| Incompatible materials | Acids. Oxidizing agents. |
| Hazardous decomposition products | No hazardous decomposition products are known. |

# 11. Toxicological information

Information on likely routes of exposure

| Inhalation | May cause irritation to the respiratory system. Prolonged inhalation may be harmful. |
| --- | --- |
| Skin contact | Causes severe skin burns. |
| Eye contact | Causes serious eye damage. |
| Ingestion | Causes digestive tract burns. |
| Symptoms related to the | Burning pain and severe corrosive skin damage. Causes serious eye damage. Symptoms may |
| physical, chemical and | include stinging, tearing, redness, swelling, and blurred vision. Permanent eye damage including |
| toxicological characteristics | blindness could result. |

Information on toxicological effects

Acute toxicity	Not known.

Components	Species	Test Results

HYPOCHLOROUS ACID, SODIUM SALT (1:1) (CAS 7681-52-9)

Acute

Oral

| LD50 | Rat | 8.91 g/kg |
| --- | --- | --- |
| Skin corrosion/irritation | Causes severe skin burns and eye damage. |  |
| Serious eye damage/eye irritation | Causes serious eye damage. |  |

Respiratory or skin sensitization

Respiratory sensitization	Due to partial or complete lack of data the classification is not possible.

Skin sensitization	Due to partial or complete lack of data the classification is not possible. Germ cell mutagenicity	Due to partial or complete lack of data the classification is not possible. Carcinogenicity	Due to partial or complete lack of data the classification is not possible.

'ARC Monographs. Overall Evaluation of Carcinogenicity

Not listed.

OSHA Specifically Regulated Substances (29 CFR 1910.1001-1053) Not listed.

US. National Toxicology Program (NT P) Report on Carcinogens

Not listed.

| Reproductive toxicity Specific target organ toxicity single exposure Specific target organ toxicity repeated exposure Aspiration hazard Chronic effects | Due to partial or complete lack of data the classification is not possible. Due to partial or complete lack of data the classification is not possible. Due to partial or complete lack of data the classification is not possible. Due to partial or complete lack of data the classification is not possible. Prolonged inhalation may be harmful. |
| --- | --- |

# 12. Ecological information

Ecotoxicity	The product is not classified as environmentally hazardous. However, this does not exclude the possibility that large or frequent spills can have a harmful or damaging effect on the environment.

Components	Species	Test Results

HYPOCHLOROUS ACID, SODIUM SALT (1:1) (CAS 7681-52-9)

Aquatic

Acute

| Crustacea | EC50 | Water flea (Ceriodaphnia dubia) | 0.1 1 - 0.13 mg/l, 24 hours |
| --- | --- | --- | --- |
|  | LC50 | Calanoid copepod (Acartia tonsa) | 0.313 - 0.497 mg/l, 24 hours |
|  |  | Coon stripe shrimp (Pandalus danae) | 0.246 - 0.333 mg/l, 96 hours |
|  |  | Daggerblade grass shrimp (Palaemonetes pugio) | 46 - <= 58.8 mg/l, 96 hours |
|  |  | Opossum shrimp (Americamysis bahia) | 0.229 - 0.321 mg/l, 96 hours |
|  |  | Rotifer (Brachionus calyciflorus) | 0.35 - 0.39 mg/l, 24 hours |
|  |  | Scud (Hyalella azteca) | 0.252 - 0.362 mg/l, 96 hours |
|  |  | Water flea (Ceriodaphnia dubia) | 0.14 mg/l, 24 hours |
|  |  | Water flea (Daphnia magna) | 0.045 - 0.068 mg/l, 48 hours |
|  | LC50 | Bleak (Alburnus alburnus) | 30 - 35 mg/l, 96 hours |
|  |  | Brown trout (Salmo trutta) | 0.02 - 0.05 mg/l, 1.5 hours |
|  |  | California grunion (Leuresthes tenuis) | 0.139 mg/l, 24 hours |
|  |  | Carp (Cyprinus carpio) | 1.75 - 1 .89 mg/l, 160 minutes |
|  |  | Channel catfish (Ictalurus punctatus) | 0.45 mg/l, 96 hours |
|  |  | Coho salmon,silver salmon (Oncorhynchus kisutch) | 0.245 - 0.328 mg/l, 15 minutes |
|  |  | Fathead minnow (Pimephales promelas) | 4.4 - 7.1 mg/l, 24 hours |
|  |  | Freshwater drum (Aplodinotus grunniens) | 1.67 - 1 .83 mg/l, 160 minutes |
|  |  | Rainbow trout,donaldson trout (Oncorhynchus mykiss) | 0.05 - 0.071 mg/l, 96 hours > 0.03 - < 0.07 mg/l, 96 hours |
|  |  | Shiner perch (Cymatogaster aggregata) | 0.045 - 0.098 mg/l, 96 hours |
|  |  | Yellow perch (Perca flavescens) | 18 - 28.7 mg/l, 15 minutes |
|  |  | Yellowtail flounder (Pleuronectes ferrugineus) | 0.1 mg/l, 24 hours |
| Persistence and degradability | No data is available on the degradability of this product. |  |  |
| Bioaccumulative potential | No data available. |  |  |
| Mobility in soil | No data available. |  |  |
| Other adverse effects | No other adverse environmental effects (e.g. ozone depletion, photochemical ozone creation potential, endocrine disruption, global warming potential) are expected from this component. |  |  |

# 13. Disposal considerations

| Disposal instructions Local disposal regulations Hazardous waste code Waste from residues I unused products Contaminated packaging | Dispose of this material and its container to hazardous or special waste collection point. Incinerate the material under controlled conditions in an approved incinerator. Dispose of contents/container in accordance with local/regional/national/international regulations. Dispose in accordance with all applicable regulations. D002: Waste Corrosive material [pH or 2 >12.5, or corrosive to steel] The waste code should be assigned in discussion between the user, the producer and the waste disposal company. Dispose of in accordance with local regulations. Empty containers or liners may retain some product residues. This material and its container must be disposed of in a safe manner (see: Disposal instructions). Since emptied containers may retain product residue, follow label warnings even after container is emptied. Empty containers should be taken to an approved waste handling site for recycling or disposal. |
| --- | --- |

# 14. Transport information

DOT

UN number	UN1791

UN proper shipping name	HYPOCHLORITE SOLUTIONS MARINE POLLUTANT (SODIUM HYPOCHLORITE) RQ

Transport hazard class(es)

Class	8

Subsidiary risk

Packing group

Environmental hazards

Marine pollutant	No.

Special precautions for user Read safety instructions, SDS and emergency procedures before handling.

Transport information on packaging may be different from that listed. Transportation information on packaging may be different from that listed.

DOT

General information	IMDG Regulated Marine Pollutant.

# 15. Regulatory information

US federal regulations This product is a "Hazardous Chemical" as defined by the OSHA Hazard Communication Standard, 29 CFR 1910.1200.

Toxic Substances Control Act (TSCA)

TSCA Section 12(b) Export Notification (40 CFR 707, Subpt. D)

Not regulated.

CERCLA Hazardous Substance List (40 CFR 302.4)

HYPOCHLOROUS ACID, SODIUM SALT (1:1)	Listed.

(CAS 7681-52-9)

SARA 304 Emergency release notification

Not regulated.

OSHA Specifically Regulated Substances (29 CFR 1910.1001-1053)

Not listed.

Superfund Amendments and Reauthorization Act of 1986 (SARA) SARA 302 Extremely hazardous substance

Not listed.

| SARA 311/312 Hazardous chemical | Yes |
| --- | --- |
| Classified hazard | Skin corrosion or irritation |
| categories | Serious eye damage or eye irritation |

SARA 313 (TRI reporting) Not regulated.

Other federal regulations

Clean Air Act (CAA) Section 112 Hazardous Air Pollutants (HAPs) List

Not regulated.

Clean Air Act (CAA) Section 112(r) Accidental Release Prevention (40 CFR 68.130)

Not regulated.

Safe Drinking Water Act	Not regulated.

(SOWA)

US state regulations

California Proposition 65

California Safe Drinking Water and Toxic Enforcement Act of 1986 (Proposition 65): This material is not known to contain any chemicals currently listed as carcinogens or reproductive toxins. For more information go to www.P65Warnings.ca.gov.

International Inventories

| Country(s) or region | Inventory name | On inventory (yes/no)* |
| --- | --- | --- |
| Australia | Australian Inventory of Industrial Chemicals (AICIS) | Yes |
| Canada | Domestic Substances List (DSL) |  |
| Canada | Non-Domestic Substances List (NDSL) | No |
| China | Inventory of Existing Chemical Substances in China (IECSC) |  |
| Europe | European Inventory of Existing Commercial Chemical Substances (EINECS) | Yes |
| Europe | European List of Notified Chemical Substances (ELINCS) | No |
| Japan | Inventory of Existing and New Chemical Substances (ENCS) |  |
| Korea | Existing Chemicals List (ECL) | Yes |
| New Zealand | New Zealand Inventory |  |
| Philippines | Philippine Inventory of Chemicals and Chemical Substances (PICCS) | Yes |
| Taiwan | Taiwan Chemical Substance Inventory (TCSI) | Yes |

United States & Puerto Rico Toxic Substances Control Act (TSCA) Inventory

*A "Yes" indicates that all components of this product comply with the inventory requirements administered by the governing country(s)

A "No" indicates that one or more components of the product are not listed or exempt from listing on the inventory administered by the governing country(s).

# 16. Other information, including date of preparation or last revision

| Issue date | 1 1-11-2016 |
| --- | --- |
| Revision date | 09-27-2023 |
| Version # | 70 |
| HMIS@ ratings | Health: 3 Flammability: 0 Physical hazard: 0 |
| NFPA ratings | Health: 3 Flammability: 0 Instability: 0 |
| Disclaimer | While Brenntag believes the information contained herein to be accurate, Brenntag makes no representation or warranty, express or implied, regarding, and assumes no liability for, the accuracy or completeness of the information. The Buyer assumes all responsibility for handling, using and/or reselling the Product in accordance with applicable federal, state, and local law. This SDS shall not in any way limit or preclude the operation and effect of any of the provisions of Brenntag's terms and conditions of sale. |